#### CHECKLIST (EV-97 Eurostar)

#### PRE START CHECKS

S  Security; Harnesses, Hatches, Control & Trim Check

T  Throttle set idle - Choke OFF - Fuel ON

A  All clear around, Facing safe direction – BRAKE ON

M  Master & charging on

P  Propeller – “CLEAR PROP” – Lookout & start 5 secs mags off

I   Ignition (MAGS) ON – choke as reqd - start

T   Temps & Pressures

M   Mags “Dead Cut” Check

R    RADIO – ON – ATIS – CALL FOR TAXI

AT THE HOLD - PRE TAKE OFF CHECKS

CHRIS THEAKSTONE   01933 653433 / 07885 600047

# Email chris@ctaviation.freeserve.co.uk

www.ctaviation.freeserve.co.uk

Brakes ON

C  Controls – Full & free movement, correct sense

H  Harnesses & hatches secure

I  Instruments – set & serviceable

E  Engine -T’s & P’s ok, choke off, set RPM 3500

Mags – Max drop 200 rpm (max diff 120)

Check Idle, reset to 2000 rpm – recheck Ts & Ps

OIL Temp AT LEAST 50° C

F  Fuel – on & sufficient for planned flight

T  Trim & flap - set for take off

A  All clear – all around & final approach clear

T   Transponder ON set to ALT

P  Power – check full RPM achieved on take off run

ENROUTE CHECK (Every 10 minutes)

L  Location – Where are you where is the airfield

I  Instruments – T’s & P’s

F Fuel – Check quantity/Estimate endurance

E Time elapsed / Estimate time to Land

#### CIRCUIT JOINING CHECK - FREDA

#### F  Fuel check state - sufficient for go around?

#### R  Radio – Circuit  (re)oining call

#### E  Engine Ts & Ps OK

#### D  Direction of landing runway, circuit & join

#### A  Altimeter –set QFE

#### A  Airfield – Check signal square, wind & traffic

DOWNWIND CHECK -

B Brakes OFF

M Check Mags ON Both

F Fuel ON & Sufficient

I  Instruments – Ts & Ps “Green”

H Harnesses & Hatches secure

#### AFTER LANDING CHECK

#### F Flaps retracted – when clear of active runway

#### T Transponder OFF (or standby)

#### SHUTDOWN

E  Engine Ts & Ps Green – Mags check “dead cut”

R  Radio, Transponder OFF

I   Ignition OFF

M Master OFF, Charging OFF

#### PRE -STALLING

#### /MANOEUVRE CHECK HASELL

##### H  Height – Min 2000 feet AGL

## A   Airframe – Configuration, integrity

###### S  Security – harnesses & hatches, no loose objects

E  Engine – T’s & P’s, fuel level ok

L  Location – clear area / airspace, allow for drift

L  Lookout – Clear all around

#### MAYDAY x 3 G-SHMI (nature of emergency,

#### location, intentions)

#### EMERGENCY SHUT DOWN - TIFS

#### T  Throttle – Closed

#### I    Ignition (Magnetos) & master OFF

#### F  Fuel - OFF

S  Security – Harnesses & hatches

ALL THE TIME     LOOKOUT

#### EV-97 USEFUL INFORMATION

Speeds

VNE (Never Exceed)		146 mph

VA   (Turbulent Air Limit)        	100 mph

#### VFE (max Flaps extended)	  77 mph

Vs   (Stall no flaps power off)    44 mph

(Stall no flaps power on)	  37 mph

#### (Stall flaps fully extend)     36 mph

Best glide speed             	 68 mph

Climbout speed		    70 – 80  mph

Approach speed	(flapless)   	 70 mph

Approach speed (with flaps)      60 mph

Typical (4000 rpm) cruise     85-90 mph

Fast  (4800 rpm) cruise     110-115 mph

#### Engine / Power

Full power max rpm            5 mins only

Continuous high rpm limit    4800  rpm

Economic cruise                  4000  rpm

Do not let engine idle below 1400 rpm.

(Always Check Pilots Operating Handbook for confirmation of operating information)

Weight & Balance – See Overleaf

PILOT’s PROCEDURES – Pre-Flight

| TASK | Tools & Forms | Notes |
| --- | --- | --- |
| Your currency? | Licence, medical, currency valid? Are You Fit to Fly? |  |
| Weather | Met office website – TAFs & METARs – synoptic - rainfall radar - Local observations | www.metoffice.gov.uk Airfield ATIS – inflight viz 3000 m + ? |
| NOTAMs | AIS (NATS) website – obtain narrow route briefing | www.ais.org.uk |
| Navigation Planning | Current Map Airfields information Nav tools Plot flight route |  |
| Exercise Planning | Pre flight familiarisation with subject matter |  |
| Tech Log Entry & Flight Authorisation | Enter new line for planned flight(s) – get authorisation – check aircraft state (hrs flown) & defects |  |
| Booking Out | Telephone Airport 01452 857700 Ext 223 | Callsign, Type (HMNY), departing direction, PoB, Fuel state, flight time |
| Fuelling & Oil | Ensure fuelled up (correctly!) & oil top up if required | Weight & Balance (under 450kg)? |
| Preflight Check | Use checklist from PoH | See main checklist |

Weight & Balance

Aircraft empty weight = 	266 kg

Max allowable AUW =  	450 kg

Therefore useful load =	184 kg

Fuel weight = 0.72 kg per litre Mogas Unld

| Fuel Load | Max remaining load |
| --- | --- |
| 20 litres = 14.4 kg | 169.6 kg 26.6 stones |
| 30 litres = 21.6 kg | 162.4 kg 25.5 stones |
| 40 litres = 28.8 kg | 155.2 kg 24.4 stones |
| 50 litres = 36.0 kg | 148.0 kg 23.3 stones |
| 60 litres = 43.2 kg | 140.8 kg 22.2 stones |

Baggage Shelf Max. Load 15 kgs

AIRCRAFT ICAO Code = HMNY