#### CHECKLIST (Thruster TST)

AIRCRAFT ICAO Code =  TST

#### PRE START CHECKS

#### S     Check chocks/Tail secured or held by helper

Security; Harnesses, Control & Trim Check

#### T     Check & Empty Carb float bowls

#### Manual pump – to refill float bowls

A     All clear around, Facing safe direction

Set Carb chokes ON (both)

Check Ignition OFF

Throttle set idle

Pull through 4 times

Set Carb Chokes OFF

Throttle set forward of idle

M  Mags ON

P   Propeller – “CLEAR PROP”

I    Start

T    Temps – allow to warm 5 mins.

Untie/release tail/chocks when warm & idling

R    RADIO – ON – ATIS – CALL FOR TAXI

NOTE: Warm start – do not use choke

AT THE HOLD - PRE TAKE OFF CHECKS

CHRIS THEAKSTONE   01933 653433 / 07885 600047

# Email chris@ctaviation.freeserve.co.uk

www.ctaviation.freeserve.co.uk

C  Controls – Full & free movement, correct sense

H  Harnesses secure

I  Instruments – set & serviceable

E  Engine –EGT OK, engine must have run at least 5 mins

F  Fuel – on & sufficient for planned flight

T  Trim - set for take off

W  Wind speed & direction check

A  All clear – all around & final approach clear

P  Power – check full RPM achieved on take off run

ENROUTE CHECK (Every 10 minutes)

L  Location – Where are you where is the airfield

I  Instruments – T’s & P’s

F Fuel – Check quantity/Estimate endurance

E Time elapsed / Estimate time to Land

#### CIRCUIT JOINING CHECK - FREDA

#### F  Fuel check state - sufficient for go around?

#### R  Radio – Circuit  (re)oining call

#### E  Engine EGT OK

#### D  Direction of landing runway, circuit & join

#### A  Altimeter –set QFE

#### A  Airfield – Check signal square, wind & traffic

DOWNWIND CHECK -

F Fuel ON & Sufficient

I  Instruments – EGT OK

H Harnesses secure

#### SHUTDOWN

E  Engine EGT OK

R  Radio OFF

I   Ignition OFF

#### PRE -STALLING

#### /MANOEUVRE CHECK HASELL

##### H  Height –    Min 2000 feet AGL

## A   Airframe – Configuration, integrity

###### S  Security –  harnesses & hatches, no loose objects

E  Engine –    EGT, fuel level ok

L  Location – clear area / airspace, allow for drift

L  Lookout –  Clear all around

ALL THE TIME     LOOKOUT

#### MAYDAY x 3 G-MTKA (nature of emergency,  location, intentions)

#### EMERGENCY SHUT DOWN - TIFS

#### T  Throttle – Closed

#### I    Ignition (Magnetos) OFF

#### F  Fuel - OFF

S  Security – Harnesses

#### THRUSTER TST  USEFUL INFORMATION

Speeds

VNE (Never Exceed)		80 kts

VA   (Turbulent Air Limit)        	70 kts

#### Vs   (Stall power off)             35 kts

Minimum sink speed             	45 kts

Best L/D Speed                       50 kts

Climbout speed		           50 kts

Approach speed	(flapless)   	45 kts

Typical (80%) cruise                 55 kts

Economic (60%) cruise            45 kts

Fast cruise                               60 kts

Assume between 11-15 lph fuel burn depending on cruise setting

#### Engine / Power

Full power max rpm 6350

Not more than 1 min. when on ground

Continuous high rpm limit    4800 rpm

Economic cruise                  4000  rpm

Do not let engine idle below 1400 rpm.

Rate of Climb

(1st minute from S.L.)   600 fpm

Takeoff Distance

To clear 15m (50ft) @ full power 100 m

Weight & Balance – See Overleaf

Pilot’s PROCEDURES – Pre-Flight

| TASK | Tools & Forms | Notes |
| --- | --- | --- |
| Your currency? | Licence, medical, currency valid? Are You Fit to Fly? |  |
| Weather | Met office website – TAFs & METARs – synoptic - rainfall radar - Local observations | www.metoffice.gov.uk Airfield ATIS – inflight viz 3000 m + ? |
| NOTAMs | AIS (NATS) website – obtain narrow route briefing | www.ais.org.uk |
| Navigation Planning | Current Map Airfields information Nav tools Plot flight route |  |
| Exercise Planning | Pre flight familiarisation with subject matter |  |
| Tech Log Entry & Flight Authorisation | Enter new line for planned flight(s) – get authorisation – check aircraft state (hrs flown) & defects |  |
| Booking Out | Telephone Airport 01452 857700 Ext 223 | Callsign, Type (TST), departing direction, PoB, Fuel state, flight time |
| Fuelling & Oil | Ensure fuelled up (correctly!) & oil top up if required | Weight & Balance (under 380kg)? |
| Preflight Check | Use checklist from PoH | See main checklist |

Weight & Balance

Aircraft empty weight            = 150 kg

Max allowable AUW             = 380 kg

Max Pilot & Passenger load = 180 kg

(396 lbs or 28 stones 5 lbs)

MINIMUM Cockpit Load       =    55 kg

Fuel is 2-stroke 50:1 mix (2%)

e.g. 5 litres   = 100 ml oil

10 litres = 200 ml

20 litres = 400 ml

FAILURE TO MIX CORRECT 2-stroke will result in engine damage or failure!

Always put oil in container first, then mix small quantity of fuel to dilute /agitate before filling remainder of container.

If in doubt ask an instructor!

Fuel = 0.72 kg per litre Mogas Unleaded

| Fuel Load | Max remaining load |
| --- | --- |
| 20 litres = 14.4 kg | 180 kg |
| 30 litres = 21.6 kg | 180 kg |
| 40 litres = 28.8 kg | 180 kg |

CONDITIONS FOR FLYING TST

Always Check Pilots Operating Handbook for confirmation of operating information

This aircraft has an air-cooled, 2-stroke engine and a single ignition system.

You are flying it in the knowledge that, statistically, there is a greater chance of an engine failure occurring than in aircraft with 4-stroke engines and dual ignition systems.

Be aware that in the event of an engine failure, the TST, being a slow, draggy airframe, will slow down quickly and appreciably.  It is essential to lower the nose quickly to keep up a safe flying speed (45 kts).

You are advised NOT to adopt a slow climbout speed because of these characteristics.

The aircraft has no braking systems.  You must keep this in mind when taxying, especially on hard airfield surfaces.