| Scenario | You call | They Reply | You Reply |
| --- | --- | --- | --- |
| Startup | (Listen to ATIS & note settings) |  |  |
| Taxi | Gloucester Tower this is G-SHMI, at the Skytime Hangar, request (radio check and) taxi, we have information ____________ & QNH set _____________ | G-(SH)MI (Read you strength _), taxi to ___________ via ______________, clear to cross runway(s) _________________ | (Read you strength _), taxi to _________ via ______________, clear to cross runway(s) _________________ G-(SH)MI |
| On the way to the hold | G-(SH)MI at ________ request clearance to cross/enter runway _____ | G-(SH)MI clear to cross/enter runway ________ OR Hold at __________ | clear to cross runway _________________ G-(SH)MI OR Holding at _____ G-(SH)MI |
| At the hold (when checks complete & pointing towards runway entrance) | G-(SH)MI at ________ ready for departure | G-(SH)MI clear to takeoff runway ______ with a RH/LH turnout VFR departure OR Hold position __________ OR Clear to Enter runway (backtrack) ________ & line up | clear to takeoff runway ______ with a RH/LH turnout VFR departure G-(SH)MI OR Holding G-(SH)MI OR Clear to Enter runway (backtrack) ________ & line up G-(SH)MI |
| Departing the circuit | (Listening watch) | G-(SH)MI change frequency (or QSY) to Gloucester Approach ________ | Change to Gloucester Approach ________ G-(SH)MI |
| Rejoining | (You may want to listen to ATIS before calling to get an update) – Listen out on approach frequency for a minute to hear who’s calling & ensure you’re not cutting in on someone. |  |  |
| Rejoin call | Gloucester Approach G-SHMI | G-(SH)MI pass your message |  |
|  | G-(SH)MI is a Eurostar inbound to Gloucester. We are at ______________ (position) at _________ (height) on _______ (pressure), request rejoin. | Roger G-(SH)MI , Overhead join 2000 on the QNH ______ Runway ________ RH/LH circuit, call ____ miles ______ of the field. | Overhead join 2000 on the QNH ______ Runway ________ RH/LH circuit, call ____ miles ______ of the field., G-(SH)MI |
| Nearing field | G-(SH)MI _____ miles _______ of the field | Roger, G-(SH)MI, change frequency to Gloucester Tower ______ | change frequency to Gloucester Tower ______ G-(SH)MI |
| Overhead field | (only if instructed) G-(SH)MI overhead the field, descending dead side | Roger G-(SH)MI, report downwind | Report downwind G-(SH)MI |
| Downwind | G-(SH)MI downwind | Roger G-(SH)MI, report final you are Number ________ | report final Number ________, G-(SH)MI |
| Final | G-(SH)MI Final (contact ____ ahead) | Roger G-(SH)MI clear to land runway ______ OR Continue approach OR Go Around | Clear to land G-(SH)MI OR Continue Approach G-(SH)MI OR Going around G-(SH)MI |